Optimal Hydropower generation management

Ahmed BENSALEM1, Abdelmalik Bouhentala2,   Salah Eddine ZOUZOU2

1„Department of Electrical Engineering. Faculty of Engineering

University of hadj Lakhdar, 05000 Batna. ALGERIA

Tel: +213–33–927287, Fax: +213–33–815123 Email:  a_bensalem_dz@yahoo.com

1,2„LEG Laboratory, Faculty of Engineering Science and Engineering

University of Mohamed Kheider, B.P 145 - 07000 Biskra. ALGERIA

Abstract: The purpose of this study is to develop a general algorithm to solve the short-term hydroelectric scheduling problem in a robust, flexible and fast way, and which retains the same performances for either a small or a large-scale problem. The solution is based on the discrete maximum principle. Gradient method is used to solve the two-point boundary value problem. To deal with difficulties posed by the state variable constraints we use the augmented Lagrangian method. This paper is particularly concerned with the handling of bonds on the state variables utilizing augmented Lagrangian method.

Key words: Short-term scheduling, potential energy, discrete maximum principle, augmented Lagrangian method.

#### 1. Introduction

The management of hydroelectric generation of power stations is very a complex problem because of the uncertainty prediction in long-term of the natural water inflows and the demand for electrical energy. A suitable method of resolution consists in dividing this problem into two sub-problems [1]:

- Stochastic long-term planning problem, the solution gives the strategic behaviour of the system. It consists in determining the total amount of water to be released from each power station across each period of the planned horizon. The long term planning horizon is divided into short periods.

- Deterministic short term planning problem, it consists in allocating the quantity of water between hydroelectric plants across the planning horizon, which is preliminary selected according to the stochastic problem. In this case, the natural water inflows and the demand for electric power are known beforehand.

The short-term hydro scheduling problem is a determinist problem [1,2], which consists of choosing the preliminary quantity of water selected to discharge from each hydroelectric plant of the system throughout the planning horizon, so that we should meet the electric energy demand. The prime objective here is to perform the operation with the lowest possible use of water (potential energy), while satisfying all operating constraints and avoiding spilling.

In this paper, we are concerned with the treatment of the deterministic problem. The discrete maximum principle is proposed for the solution, whereas the zero gradient equations representing the necessary conditions for optimality relating to the discrete maximum principle are iteratively solved by the gradient method. To deal with the two-sided inequality constraints issued from the mathematical model of the system, we use the augmented Lagrangian method. This method is a combination of two methods, that is the penalty method and the local duality method, which allows moderating the disadvantages of both methods [3].

The hydroelectric system considered in this paper consists of reservoirs in cascade located on the same river [4]. The time taken by the water to travel from one reservoir to the downstream reservoir and the water head variation are taken into account. The natural inflow and the demand for electrical energy are known beforehand. The optimization is done over one week divided into hours.

2. Problem formulation

The main objective of the short-term hydro scheduling is to select the quantity of water to be released from each hydroelectric plant across the planning horizon in order to maximize the total water stored in all reservoirs at the end of the planning horizon, while  satisfying demand for energy and all other constraints[1].

The adequate mathematical model to the short-term hydro scheduling problems can be formulated as follow:

2-1. Objective function:

The main objective is to maximize the total potential energy of water stored in all the reservoirs. The formulation must take into account the fact that the water stored in one reservoir will be used in all its downstream reservoirs, hence, the water stored in the upstream reservoir is more valuable than that stored in the downstream reservoir [1]:

Where:

: Last period of the planning horizon.

: Potential energy of water stored in reservoir  at the end of the planning horizon. This energy depends on the amount of water stored in the reservoir , on its effective water head and on the effective water head of the downstream reservoirs.

Total potential energy of the released water  from reservoir , which will reach later the downstream reservoir  after the last period of the planning horizon.

: The reservoir immediately preceding the reservoir .

Smi: Time required for the water discharged from reservoir  to reach its direct downstream reservoir, in hours.

2-2. Operational constraints:

The principal operational constraints of the system are the following [1-2, 5-8]:

- Hydraulic continuity constraint:

For each reservoir at each time period , the following constraint establishes the water balance equation:

Where:

: Content of the reservoir  at period , in Mm3.

uik: Turbine discharge of hydroelectric plant  in period , in Mm3.

: Spillage of hydroelectric plant in period , in Mm3.

: Total inflows to reservoir in period , in Mm3.

Taking into account hydraulic coupling, the total inflow to reservoirs is determined as follows:

: Natural inflows to reservoir in period , in Mm3.

: Turbine discharge from hydroelectric plant , which will reach later the downstream reservoir  at period .

- Minimum and maximum storage limits:

xi≤xik≤xi

: lower and upper limits on reservoir storage capacity, respectively, for reservoir, in Mm3.

- Minimum and maximum discharge limits:

ui≤uik≤ui

: Minimum and maximum limits on water discharge, respectively, of hydroelectric plant.

- Demand-generation balance:

The total power generated by all the hydroelectric plants must satisfy the system load demand at each period of the planning horizon. In mathematical terms, this has the following form:

i=1nPik=Dk

Where:

Dk: The demand for electrical power at each period , in Mw.

Pik: Electric power generated by hydro plant  at period , in Mw.

: Number of reservoirs of the system.

3. Solution method

In mathematical terms, we formulate the short term scheduling problem of a hydroelectric plant system as follows [1]:

(1)

Subject to the following constraints:

(2)

i=1nPik=Dk                                	               (3)

(4)

(5)

(6)

This problem can be solved by applying the discrete maximum principle as follows [1, 9-10]:

Associate the constraint (2) to the criterion (1) with the dual variableλik.To satisfy the balance between electric energy demand and generation we associate the constraint (3) to the criterion (1) with the Lagrange multiplierβk, and then we define the functionHk, called the Hamiltonian function, which has the following form:

The problem (1)-(6) becomes:

(7)

Subject to constraints (4)-(6), with the conversion equation of the associate variable [9]:

λik−1=¶Hk¶xik−1                                                  (8)

When constraints (4), (5) and (6) are inactive, the optimal trajectory uik will be reached when the following optimality conditions for each hydroelectric plant and at each period are satisfied:

¶Hk¶uik=0                                                            (9)

In order to solve these equations, we needs to know the boundary conditions, which are:

- The first one is the initial state, which is specified, i.e., at the initial time, the initial content of all reservoirs are known, thus:

(10)

- The second one is the terminal condition for the adjoint equation:

(11)

Consequently, equations (2) and (8)-(11) constitute a two-point boundary value problem whose solution determines the optimal state and control variables. This problem is solved iteratively by using the gradient method.

To take into account possible violations of constraints (4) and (5) we proceed respectively as follows:

- If the value of some uikwhich satisfies the optimality condition (8) violates the constraint (5), we’ll fix them to their boundary limits, the others are left free. Then a new research for the optimum is made but with only the free variables.

- To deal with possible violations of constraint (4), we use the augmented Lagrange method [1,3,11], which consists in adding a function Rik to the HamiltonianHk, which penalizes the violation of constraints (5). Then the Hamiltonian Hkbecomes:

(12)

The function Rik is defined as follows:

Rik=ρikYik+r(Yik)2                                    (13)

Where:

: Penalty weight.

ρik: Lagrange multipliers, updated as follows:

ρik=ρik+2r max(xik−xi,−ρik2r)                  (14)

The function  is determined as follows:

Yik=max(xik−xi,−ρik2r)                               (15)

4. Application

In order to testify the efficiency and the performances of the method we apply the proposed algorithm to the system composed of four reservoirs in cascade located on the same river as shown in Fig.1 [4]. The characteristics of the system reservoirs and water time travel are shown in table 1:

Fig.1: The power system configuration

| i | xi (M.m3) | ui (M.m3/h) | Ci | (h) |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | 550 145 5,1 424 | 0,000 1,969 2,138 4,248 | 0,0000 2,4525 2,4525 2,4525 | 3 2 5 - - - |

Table 1: Characteristics of the installations.

Where:

xi: Maximal storage capacity of reservoir .

ui : Maximal discharge capacity of reservoir .

: Time taken by the water to travel from reservoir   to reservoir .

The following relation determines the water head [4]:

Where:

Discharges

: Positive constants characterizing the reservoir .

The constant values of each reservoir are given in the following table 2:

| i | hi' (m) | hi'' (m) | αi |
| --- | --- | --- | --- |
| 1 2 3 4 | 0 194 88 174,4 | 0 195 88,01 199 | 0 0,0614 0,8627 0,0074 |

Table 2: Characteristics of water head.

The natural flows  are assumed constant throughout the week at the four sites. Their values are given in table 2 as well as the initial content  of each reservoir of the system:

|  | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| (M.m3/h) | 0,6228 | 0,2304 | 0,2196 | 0,9576 |
| (M.m3) | 225 | 18 | 2,5 | 212 |

Table 3: initial contents and the natural inflows

The demand for electrical power Dk is shown in figure 2.

Fig. 2: Weekly demand for energy

#### 5. Interpretation

The optimal solution (Fig.3) is obtained while satisfying all operating constraints after a very moderate number of iteration compared to that obtained by A. Turgeon [4] for the same system.

Period

Fig.3: Optimal discharge trajectories.

We can reduce this number by using an optimal step in the gradient method of our algorithm, and by adjusting adequately the penalty weight factor . This will lead to a rapid exit from the violated zone as it is shown in table n° 4.

| Penalty weight | 1 | 5 | 10 |
| --- | --- | --- | --- |
| Number of iteration | 100 | 88 | 81 |

Table 4: Influence of penalty weight on iteration.

The optimal trajectory solution obtained is shown in Fig.3. We state on one hand that the discharge follows the demand because the production is proportional to the discharge and must be equal to the demand. On the other hand, the discharge in the downstream reservoir is greater than the discharge in the upstream one, because the water stored in the upstream reservoir is more valuable than the water stored in the upstream one, i.e., the water of the upstream reservoir will also be used in the upstream ones. Consequently, in economic term water in the upstream reservoirs should be preserved as shown in Fig.3.  Consequently, the reserve in potential energy of the system becomes more important, in spite of the considerable decrease in the contents of the downstream reservoirs. In addition, management is done without the draining of any reservoir as it is shown in table n° 5.

|  | 1 | 2 | 3 | 4 | Total |
| --- | --- | --- | --- | --- | --- |
| initial content (Mm3) | 225 | 18 | 2,5 | 212 | 457,5 |
| final content (Mm3) | 283 | 0,3 | 0,2 | 180 | 463,5 |

Table 5: initial contents and final contents.

The optimal management of hydroelectric system allows having the final contents of all the reservoirs higher than the initial contents as it is illustrated in table n° 5.

The behavior of the objective function during the optimization process is illustrated in fig.4. We note that the search for optimum is very slow when we approach the solution; this is due to the gradient method it self.

Potential energy

Iteration

Fig.4: Behavior of the search trajectory (objective function).

6. Conclusion

With the discrete maximum principle, the optimal solution is obtained by solving simultaneous equations representing the optimality conditions. It turned out to be efficient. And so was the augmented Lagrangian method. Hence the algorithm proposed requires moderate time and storage for its execution and can be applied efficiently to large dimensions problems.

More improvements could be made to the proposed algorithm in order to increase the speed convergence of the algorithm and its execution time by improving the gradient method, and by adjusting adequately the penalty weight factor.

Taking into account the time delay between reservoirs increases considerably the dimension of the problem.

7. References

- Bensalem A., Turgeon A. : Gestion Optimale d’un système Hydroélectrique à court terme (Optimal short term hydro scheduling). In Proceedings of the 1th International Conference on Renewable Energy’99, November 23-24, 1999 Tlemcen. Algeria.

- Escudero LF, Fuente JL, Garcia C.  Prieto FJ. : Hydropower generation management under uncertainty via scenario analysis and parallel computation. IEEE Trans PS 1996; May 1966, vol 11, n° 2 p.683-689.

# Bertsekas D.P.: Multiplier Methods: A Survey. Automatica, Pergamon Press, 1976. Printed in Great Britain, Vol. 12, p. 135-145.

- Turgeon A.: Optimal short-term hydro scheduling from the principle of progressive optimality. Water resources research, June 81, Vol. 17, n°3, p. 481-486.

- Naresh R. Shatma J.: Hydro System Scheduling Using ANN Approach. , IEEE Transactions on Power Systems. vol. 15 n° 1, Feb. 2000, p. 383-395.

- Lecler G.: La Programmation Linéaire et ses Applications en Gestion des Systèmes Hydriques (Linear programming and their applications for the management of hydro systems). Conference presented in workshop: analysis of risk in the management of hydro systems.  Polytechnic of Montréal, November 19-28, 1985. Montreal.

- Fosso O. B., Belsnes M.M.: Short-term Hydro Scheduling Liberalized Power System. In: Proceedings of the International Conference on Power System Technology POWERCON ’04, Nov. 21-24, 2004, Singapore, p. 1321-1326.

- Ray W.H., Soleman M.A: The optimal control of processes containing pure time delays. Eleventh joint automatic conference of the America Automatic Control Council, 1970. p. 473-483.

- Sage A.P.: Optimum system control. John Wiley & sons Inc. New York 1968.

- Oh Y.N.: An application of the discrete maximum principle to the most economical power-system operation. Electrical Engineering in Japan, 1967, Japan, Vol. 87, p. 17-29.

- Pierre D., Lowe M.J.: Mathematical Programming via Augmented Lagrangian. Addison Wesley publishing company 1975.