A COMPARATIVE INVISTIGATION OF ANFIS PARAMETERS FOR THE DEVELOPMENT OF AN ADAPTIVE FUZZY LOGIC SPEED CONTROLLERS FOR AC DRIVES

# M. Nour1, I. Aris2, N. Mariun2, and S. Mahmoud2

1 School of Electrical and Electronic Engineering, University of Nottingham Malaysia Campus,

Jalan Broga, 43500 Semenyih, Malaysia, mutasim.nour@nottingham.edu.my

2 Department of Electrical and Electronic Engineering, University Putra Malaysia

43400 Serdang, Selangor, Malaysia

Abstract: Despite the extensive work carried out on developing fuzzy logic controllers (FLC) that is either more intelligent/adaptive or employs fewer rules, there seems to be a lacking of a comprehensive explanation on how the proposed FLC scheme came about as most claimed that it was by trial and error and /or experience. This paper attempts to fill in this gap, by providing an in-depth explanation on how the Adaptive Neuro Fuzzy Inference System (ANFIS) can be used to develop FLC that are not only adaptive, but also use as few as 15 rules. This is achieved by investigating and understanding the ANFIS parameters involved and taking into consideration how each parameter affects the resulting speed control performance of the FLC. The ANFIS parameters studied are the type and range of training data, initial parameters of the FLC architecture and training method that should be used to train a FLC to be adaptive. The simulations were carried out in the MATLAB® environment.

Key words: PMSM, Adaptive Speed Controller, ANFIS, Neural Network, Fuzzy Logic Control, AC Drives

### 1. 	Introduction

Though conventional PI controllers have been widely utilized as speed controllers for high performance ac motor drives (HPACMD), the fixed gains of these types of controllers are responsible for making these drive systems very sensitive to motor parameter variations, load torque and speed disturbances [1-2, 5-6]. HPACMD require fast and accurate speed response, quick recovery of speed from any disturbances and sensitivity to motor parameter variations [2, 3]. For the motor drive to be capable of continuously running at its desired response under different operating conditions such as speed command tracking, load torque disturbances, and parameter variation, the speed controller has to be capable of self-tuning, or adapting its gains automatically online in response to these different operating conditions. Such controllers are commonly known as adaptive controllers.

Although there are many types of adaptive control techniques that exist to assist control designers to develop adaptive speed controllers, their control principle falls generally in either one of the two classifications of adaptive control techniques; Plant or Model-based or Artificial Intelligence (AI)–based [1]. Self-Tuning Control, Model Referencing Adaptive Control (MRAC) and Sliding Mode (or Variable Structure) Control and Adaptive Back Stepping Control are examples of adaptive control techniques that are based on the mathematical model of the plant  [1, 8, 17, 18]. Adaptive control techniques such as the Expert System Control, Fuzzy Control, Neural Control, and Genetic Algorithm Control are referred to as AI-based techniques since they embed the experience and intuition of the human plant operator, designer and/or researcher into the system [1, 6]. These control methods are also known as intelligent techniques as they are designed to have the computing capability to learn, self-organise and/or self-adapt their parameters to enable the plant’s response to be robust towards machine parameters and load torque variations [1, 6].

Recent literature has paid much attention to the potential of fuzzy control in machine drive applications [4-5]. FLC has the advantages of providing robust performance for both linear and non-linear plant functions. Also FLC is a model free estimator which does not require knowledge of the plant’s mathematical model [1-2, 14]. However, the qualitative design of the FLC is entirely heuristic, and thus difficult to obtain a systematic design as it is based on the experience and expert knowledge of the process being controlled [4, 13]. Besides being subjective and heuristic, the determination of its fuzzy rules, input and output scaling factors, and the choice of membership functions depend on trial and error that makes the design of the FLC a time-consuming task [3-4]. To overcoming the disadvantages of the FLC, some efforts have been made on the use of neural networks (NNs) to design the FLC [1, 3, 6, 10, 19]. This neural fuzzy approach is a form of a hybrid intelligent system that combines at least two intelligent technologies, such as those that were mentioned earlier [6].

The NN gives the FLC the ability to learn systematically from a given set of training data instead of resorting to time-consuming trial and error methods. Among the advantages [1, 6, 9-12] of the ANN is that it has fast performance due to its massive parallel input-parallel output multidimensional computing system and distributive computation. The network architecture is inherent to noise and is also fault tolerant due to its distributed knowledge/intelligence throughout the network. This makes the network virtually unaffected by a few erroneous weights or destroyed connections. However, if the model is not trained with sufficient accuracy, the stand alone neural model may not be stable as was encountered by the authors of [3]. An important type of neural fuzzy system is the ANFIS, which is the topic of discussion of this paper.

Encoder

FLC controller

dq

abc

Hyestersis Current Controller



Diode Rectifier

3 Phase Inverter

PMSM

Vac

d/dt





*

iq*

id* =0

ia  ib  ic

ia*

ic*

ib*

PWM

Adaptive Neuro Fuzzy Inference Algorithm

E

CE

iq

d/dt

CE

E

The rest of this paper is a compilation of the results and conclusions of the findings and observations that were collected over the duration of the development of the adaptive control algorithm known as Function Torque Adapted Gains Fuzzy Inference System (FTAG FIS). The FTAG FIS was developed using the MATLAB® Fuzzy Logic Toolbox ANFIS Editor GUI for a HPACMD. The design of the proposed controller incorporates a function known as FTAG that automatically adjusts the output gain of the FLC according to the load applied onto the motor, effectively eliminating the need for the control designer to adjust the input-output gains of the FLC manually by trial and error. The FTAG FIS algorithm guarantees the closed loop performance of the motor even when the motor is subjected to significant and unpredictable motor parameter variations and load torque disturbances at both low and high speeds. An outstanding advantage of the FTAG FIS algorithm is that it used only 15 rules in its rule base, which is less than half the number of rules that is being used in many other FLC that has been proposed in other papers such as in [4-5, 13-14].

However, though there are many papers written on the many different types of adaptive control techniques employed to develop HPACMD speed controllers, the focus of most of them are to relay details such as how the developed controller works and how it improves the performance of the motor compared with other types of controllers. There seems to be a lacking of papers that give an account of reasoning, explanations, findings and observations that have led to the development of the proposed control algorithm. There are even less papers on adaptive controllers for HPACMD that were developed using ANFIS. Books such as [1, 6, 9-10] concentrate on explaining the concept of how ANFIS works rather than how it can be used to develop an adaptive speed controller for HPACMD. This paper seeks to fill these gaps.

Fig.1. Adaptive Neuro Fuzzy Control Schematic Diagram

2. Adaptive Neuro Fuzzy Control Scheme

The vector controlled PMSM drive control system used in this work is shown in Fig. 1. The fuzzy logic speed controller is generated from scratch using ANFIS. The training data are generated from the drive system using a PI as an expert speed controller. The PI gains are adjusted for different operating conditions to achieve good transient and steady sate responses. The inputs to the ANFIS algorithm are the speed error E and change of speed CE while the output is the change of the torque current command iq

- Fuzzy Logic Controller (FLC)

The FLC used in this work is the well-known PI type controller [1,4,5]. Its inputs are the speed error E and change in the speed error CE, while its output is the change in the torque current command iq. A typical FLC using zero order Sugeno inference is shown in Fig. 2. The operation of this FLC can be summarised in the following steps;

The first step is the fuzzification, which is the mapping of the crisp input variables E and CE into fuzzy sets, which are referred to as membership functions of the fuzzy variables. A membership function (MF) is a curve that defines how the values of a fuzzy variable in a certain region are mapped to a membership value between 0 and 1 [1,6]. The MFs can be triangle, trapezoidal, bell, Gaussian, or any other polynomial based curves.

1

-6

0

6

µ(E)

NS

1

-6

0

6

µ(CE)

ZE

1

-6

0

6

µ(iq)

K1

0.8

0.6

DOF1 = 0.6

1

-6

0

6

µ(E)

1

-6

0

6

µ(CE)

ZE

1

-6

0

6

µ(iq)

K2

0

0.6

DOF2 = 0

ZE

1

-6

0

6

µ(E)

NS

1

-6

0

6

µ(CE)

PS

1

-6

0

6

µ(iq)

K3

0.8

1.0

DOF3 = 0.8

E

CE

Rule- 1

Rule- 2

Rule- 3

Z

Fig. 2. A two input, three rule fuzzy system using Sugeno zero order method.

The second step is the application of the AND fuzzy operator in the IF (antecedent) part of the rule. When applying the AND operator in the rule, the degree of fulfilment of rule n (DOFn) will be either the minimum or the product of the two degree of memberships (DOMs) μn(E) and μn(CE) of that rule. The third step is the implication from the antecedent to the consequent (THEN part of the rule). This is defined as shaping the output fuzzy set based on the antecedent. The input of the implication processor is a single number given by the antecedent and the output is a shaped fuzzy set. As the ANFIS is based on the Sugeno type inference model, only the Sugeno implication method is considered here. The Sugeno method can be defined in two ways: the zero order or the first order method. In the Sugeno zero order method, the fuzzy output MFs are only constants; as in the fuzzy output, Zn = constant, Kn, as shown in Fig. 2. In the Sugeno first order method, the fuzzy output MFs have linear relations with the inputs; as in the fuzzy output, Zn, is mathematically expressed as,

(1)

where all the A’s are constants. The fourth step is the aggregation of the consequents across all the rules [1,6], where the total fuzzy output, Zf, is the union of all individual fuzzy outputs, Zn. In this step all the shaped output fuzzy sets are united into one fuzzy set. The final stage in the FLC is the defuzzification of the united fuzzy output, Zf. The input to this process is the aggregation of all the fuzzy output and the output is crisp number, which is the change of the torque current command. In this work, the weighted average defuzzification method is used due to its computational simplicity [6].

- Adaptive Neuro Fuzzy Inference System (ANFIS)

In this process, the entire FLC is generated and trained using artificial neural networks (ANN). A simplified schematic of the ANN is shown in Fig. 3. An ANN is a model that is made to learn by example from the sets of input output training data fed into it. The ANN results from the interconnection of artificial neurons via weights that act like gains. A non-linear activation function contributes to the non-linear transfer characteristics of a neuron, which permits non-linear input output mapping in a NN. Once the appropriate NN model has been designed and/or selected, the network is trained to give out the corresponding desired patterns at the output; this can be accomplished offline or online using either the supervised, unsupervised/self or reinforced learning/training method [11].

The learning process of the network has been illustrated simply in Fig. 2 [11], where the values of the weights are adjusted by the training algorithm until the network output matches the target. The training algorithm is usually a function of the error between the desired output and the NN output. The training continues until the error between the target and network output has reached zero, or to an acceptable low level.

Compare

Neural Network including connections (Weights) between Neurons

Target

Inputs

OutputNN

Adjust Weights

Error

Fig. 3. Simplified schematic of the ANN training process.

The learning process of the FIS using ANFIS is illustrated in Fig. 4 [1,6], where the parameters of the MFs and Sugeno output functions, f1 and f2, are adjusted by the Backpropagation (BP) algorithm until the FIS output matches the target.

Input

A1

A2

B1

B2

П

П

N

N

П

П

Σ

Backpropagation algorithm

CE

E

iq

Target

Fd

μA1(E)

μA2(E)

μB1(CE)

μB2(CE)

W1

W1

W2

W2

AND

Normaliser

W1

W2

W1f1

W2f2

ξ

Z f2

MFs

Z f1

-_

+

Layer 1

Layer 2

Layer 3

Layer 4

Layer 5



MFs

Fig. 4. ANFIS structure for a Sugeno fuzzy inference system.

The corresponding ANFIS network shown in Fig. 4 consists of the following five layers:

Layer 1 is the fuzzification layer, where the neurons in this layer perform fuzzification. The subscript numbers 1 and 2 refer to the rule number. The outputs of this layer are the DOMs of each input with respect to the MFs it has been mapped to. Layer 2 is the rule base layer, where each neuron corresponds to one rule in the rule base. The outputs W1 and W2 are the DOF for the first and second rule respectively, which are then normalized in Layer 3. In layer 3, the ratio of the DOF of one rule to the sum of all the DOFs from each rule is calculated out to determine the overall contribution of that particular rule to the final result, F. The function of Layer 3 can be expressed mathematically as,

Wi=WiW1+W2 i=1, 2			(2)

Layer 4 and 5 are the defuzzification and summation layer respectively. They perform the calculation and summation of the output crisp value to determine the overall ANFIS output, F (iq). In this case, Zn = fn and DOFn = Wn. The calculated output F of the FIS network is then compared with the desired output, Fd, which has been generated as the input-output, training data pairs. The error  between them is fed into the BP training algorithm which trains and adjusts the parameters of the FIS. In this work, the constants of the Sugeno fuzzy output membership functions f1 and f2, and the parameters (shape and distribution) of the input MFs are made adaptive.

The ANFIS training of the FIS continues either until the error  equals the specified error tolerance (which is usually left at 0) or when the specified number of epochs has been reached, whichever comes first. One epoch refers to one training round; in one training round, the ANFIS passes all the training data pairs through the FIS architecture that has been loaded into it, and then adjusts the parameters of the FIS. A FIS has been trained successfully when it is able to generalise its response for all types of inputs apart from those that it has been trained with.

# 3. 	Analysis of Simulated Speed Responses

The ANFIS training were carried out using the MATLAB® ANFIS Editor GUI. A more comprehensive explanation on the use of the MATLAB® ANFIS Editor GUI is provided in [12]. The training data were generated using the PI speed controller of the MATLAB® Simulink control drive scheme model shown in Fig. 1. The real parameters of the PMSM used are as follows: 370W, 230V AC, 8 poles, 6000 rpm, 0.6 Nm rated torque and 0.25 kgcm2 motor inertia. Before developing the adaptive speed control algorithm for the dynamic model of the motor, it was developed initially on its reference model where only the motor speed dynamics are only considered. This was based on the theoretical reasoning that a vector controlled, dynamic model of a HPACMD can be controlled as how one would with a separately excited dc motor drive. Hence, the developed speed control algorithm of the reference model should be a good approximate to that which is required by its dynamic counterpart. Therefore, in speed control design, it is irrelevant whether the dynamic machine is a dc machine or a vector controlled ac machine, the results of the study are universally applicable to all HPACMD, as long as the speed controller is under consideration [15]. Therefore, extensive experimentations were carried out to determine the best method that generates the most appropriate training data from which the FIS can learn from, the best initial FIS architecture parameters that has the best learning ability, and the most appropriate training method to train the FIS. These issues are addressed in the following sections.

- Training Data

The training data acts as the learning material that ANFIS uses to train the FIS. Thus, it is imperative to understand how aspects of the training data such as its type, number, way of distribution and range influence the ANFIS training result.

- Type of Training Data

One of the main concerns with developing an adaptive speed controller using ANFIS is whether the training data that was generated at one particular operating condition of the motor would be sufficient enough to train the FIS to effectively control the speed of the motor at its other operating conditions. In other words, is the generated data sufficient enough for the FIS to generalise for all the other changes in the operating conditions besides those at which the training data was generated from?

Inductance, L

Inertia- Friction, J&B

Origin

Node 1

Resistance, R

Fig. 5. A graphical representation of the different operating points of the motor parameters during running, assuming rated load.

The changes in the operating conditions due to changes in the motor parameters can be explained by referring to Fig. 5. The motor may operate at any of the points that make up the volume of the cube. The origin refers to the rated values of the motor parameters R, L, B and J. on the other hand, node 1, represents the maximum expected variation of inertia Jmax, friction factor Bmax, resistance Rmax, and inductance Lmax. Therefore, all the expected parameters variation of the motor and drive lie inside the cube. In addition, the load variation and disbranches must be included as variation in the system operating conditions. An intelligent adaptive speed controller has to be able to adapt to all possible load conditions as well. Fig. 6(a) shows that the speed control performance of the FIS that was trained at rated motor parameters (Jrated and Brated) and at No Load (NL) condition, deteriorated at FL condition. Does this mean that one has to train the FIS with training data that is generated from all the possible operating conditions of the motor for it to learn to be adaptive?

The trained FIS will not know how to respond to situations that it has not been trained for, but it can be trained with data that has been generated at a few prime situations that may be able to help it generalise for other untrained situations. For example, a FIS was trained with training data that was generated at the two extreme ends of the motor’s operable and load application; NL and FL condition. The FIS, however, was not trained specifically for different motor parameters (such as J and B) besides rated values because the results in Fig. 6(b) indicated that the trained FIS was able to adapt to large changes in the motor parameters relatively better than to changes in the load. By this method of training, it was aimed that the trained FIS would not only be able to adapt to the motor’s NL and FL condition, but also to the other loads in between that it was not specifically trained for.

Speed, N (rpm)

Time, t (s)

(a)

Speed, N (rpm)

Time, t (s)

(b)

Fig. 6. Comparison of the speed control performance of the ANFIS trained FIS between (a) FL and NL motor operation and (b) rated and maximum motor parameters.

Speed, N (rpm)

Time, t (s)

NL

HL

FL

Fig. 7. Comparing the speed control performance between a FIS that has been trained for NL condition only, and a FIS that has been trained at NL and FL condition.

The results in Fig. 7 verify the effectiveness of training a FIS at two ‘extreme’ load conditions, NL and FL, i.e. the minimum and maximum load conditions that the motor can operate at. This is an important finding as this goes to show that the FIS does not need to be trained for every load and parameter condition that the motor faces, and that training the FIS at NL and FL (and rated motor parameters) is sufficient for the FIS to generalise for all other load conditions.

- Number of Training Data

Based on the previous finding, it would seem that the next obvious thing to do was to generate more training data from other load conditions to further improve the control performance. However, another concern regarding training data is the amount of training data that is allowable to train the FIS. In [16], it is highlighted that determining the ‘right’ number of training data to train is frequently a process of trial and error. With too many data, the FIS would simply memorise the data (no generalisation), but on the other hand, with too few data, the FIS would be prevented from learning the relationships among the variables. Not only that, the amount of training data is also dependent on the FIS architecture [12, 16]. Though there are a few heuristics in estimating the number of training data to use, the design criteria suggested in [12] was abided, where the number of training data pairs should be several times larger than the number of FIS parameters being estimated. Naturally, the FIS parameters increase as the number of MFs increases. Though the ratio between data and parameters varies from case to case, a ratio of about five was used as illustrated in an ANFIS training example in [12].

To help determine whether this chosen ratio was appropriate, a comparison was made between the speed control performance of a FIS whose input was defined by five MFs and another FIS that was defined by nine. Though the latter has more FIS parameters to be estimated by ANFIS, both were trained with the same number of training data. It was found that the control performance of the five MF FIS was generally far better than the nine MF FIS, regardless of the input output gains used. Theoretically, using nine MFs should give better control performance than five since there are more rules and more control parameters involved. However, when the nine MF FIS was retrained with five times more training data than the five MF FIS, it was found that it performed better than the five MF FIS, especially in its percentage over and undershoot during load changes at low speeds. Hence, in order to attain a justified comparison between any ANFIS trained FIS, the number of training data used must be in proportion to the number of FIS parameters being estimated by the ANFIS.

It is desired not to resort to using more training data with more MFs per input because more training data would require longer training time, which can span anything from 10 minutes to a few days, depending on the number of data available, even with a high performance PC. Besides that, reducing the number of MFs in the control algorithm will reduce the computation time during real time implementation.

- Distribution of Training Data

Another concern with regards to the training data is the manner in which the data is distributed, or arranged, as it is passed through the ANFIS to train the FIS. For example, does it matter if all the NL training data type was passed through ANFIS first before the FL data type? Would this result in a FIS that may perform better at one condition than the other? This was verified by comparing the speed control performance of a FIS that was trained with ‘sequenced’ data with one that was trained with random data. ‘Sequenced’ here implies that all the NL data were passed through first before the FL data, whereas random implies that the NL and FL data were passed through in random order. From this, it was found that it does not matter whether the different types of training data pairs were passed through the ANFIS in random or sequence order as both FIS had similar performance.

- Range of Training Data

The control performance of several FIS that trained with data generated at different ranges of speed changes was also compared to see the amount of influence that it had on the training performance of the FIS. In the initial FIS trainings, random step speed command changes between 6000 rpm were applied to the motor. Then the speed error, E, and command current, Iq*, were generated from 0-6000 rpm speed changes because hypothetically, the FIS should be able to generalise for the reverse motoring (negative speed commands) mode of operation. During the earlier part of the development of the FTAG FIS, it was assumed that by narrowing the range of the random step speed command changes onto the motor would result in smaller training error, and thus, better FIS control performance. For example, a FIS that was trained with a set of training data that fell within 1 rpm performed better than the one that was trained with 0-6000 rpm data. However, it is found that the FIS should be trained with data that will allow the FIS to learn about the different behaviours of the data at different speed ranges. In other words, the set of training data should consist of information that will tell the FIS how it should behave at low, mid and high speed command changes. For example, speed changes of 0 - 1 rpm, 1 - 0 rpm, 0 - 10 rpm, 10 - 0 rpm, and so forth until the rated speed were applied, and then the respective input output data that were collected were combined together. To reduce the amount of training data and hence reduce training time, redundant data such as those that are repetitive during steady state response can be removed, leaving only the data that contains information of the transient response.

- Initialisation of FIS Architecture

As with the training data, there are also a few concerns regarding the initialisation of the FIS architecture that would be used for training. Among these concerns are the number and type of MFs that should be used to define the inputs, the number of inputs and thus, the number of rules the controller should have. The initialisation of the FIS architecture is like the child being taught with that learning material presented to it by the ANFIS, which acts like the teacher in this analogy. The initial parameters of the FIS before training determine how fast or slow the FIS learns. This is covered next.

NL

0s – 0.05s

FL

0.05s – 0.1s

Time, t (s)

Speed, N (rpm)

NL

0s – 0.03s

HL

0.03s – 0.06s

FL

0.06s – 0.09s

Speed, N (rpm)

Time, t (s)

(a)

(b)

Fig. 8. Comparing the speed control performance between 9, 7, 5, and 3 MFs FIS at (a) Low Speed (5 rpm) and (b) High Speed (6000 rpm). Rated J and B.

- Number of Input MFs

The control performance of a FIS that has more MFs is generally better than one that has less. This is naturally due to the fact that each point in the input and output spaces are more narrowly mapped onto the DOM between 0 and 1 if is defined by many MFs [12]. However, it was desired to determine the least number of MFs that can be used (to define the input and output of the FIS) that will give a speed control performance that is acceptable and within limits as one that has many MFs. This is to reduce the number of MFs and rules in the adaptive speed control algorithm for easier implementation and faster execution time.

The results in Fig. 8 show that all four FIS have similar speed control performance during step changes in loads at high speeds (upon training each FIS with both NL and FL training data type). However, at low speeds, the speed control performance using seven or even nine MFs to define the input does not seem to be as good as using five MFs. This could be very much due to the fact that both the seven and nine MFs FIS were trained with the same amount of training data as the five MFs FIS; it has been highlighted in Section A.ii on the importance of training the FIS with an amount of training data that is in proportion to the number of FIS parameters being estimated by the ANFIS. However, this matter in validating this comparison was not pursued further as it was desired to retain the fewer number of MFs. As for the three MFs FIS, it was found that it still did not perform any better than seven MFs FIS even though the number of training data was reduced in proportion to its number of parameters. This could be due to the insufficient number of parameters in the FIS to learn properly. Thus, using five to define an input seems to provide the most acceptable performance that is sufficiently good enough for the purpose at that time.

Speed, N (rpm)

Time, t (s)

NL

0s –N 0.05s

FL

0.05s – 0.1s

(a)

NL

0s – 0.03s

HL

0.03s – 0.06s

FL

0.06s – 0.09s

Speed, N (rpm)

Time, t (s)

(b)

Fig. 9. Comparing the speed control performance between using linear and constant Sugeno output MFs at (a) Low Speed (5 rpm) and (b) High Speed (6000 rpm).

- Sugeno Zero or First Order Method

A speed control performance comparison was also made between a FIS that employed the Sugeno zero order method and one that employed the Sugeno first order method. The results from the comparison in Fig. 9 clearly show that the Sugeno first order method offers far better performance than the Sugeno zero order method.

- Type of MF

It was explained before that the inputs of the FIS can be defined by different types of MFs, such as triangle, trapezoidal, bell, Gaussian, or any other polynomial based curves. A number of comparisons were initially carried out between the triangle, trapezoidal, bell, and Gaussian MFs to understand and identify which MF type was most well suited to be used in an adaptive fuzzy control algorithm. One of the important comparisons that were performed was the speed control performance comparison between using triangle/trapezoidal, bell and Gaussian MFs to define the one input FIS that underwent the ANFIS training. From here, it was found that the performance of the FIS that uses either triangle/ trapezoidal or Gaussian MF curves was very much dependent on the input and output gains that were used and the operating condition of the motor it was running at. For example, the same gain that was used in one operating mode cannot be used in another mode, whereas in some speed ranges, it was unable to track the command speed at all. This was not the case for the FIS that had used bell MFs curves. The bell type MF seems to provide the most robust speed performance that could be attributed to its characteristics that ‘is continuous, differential and always has some finite value’ [1]. However, this is inconclusive as it also depends on the parameters of the training data that were used to train the FIS, which has been covered in Section 3A. For simplicity, It was resorted back to using the more commonly used trapezoidal MFs to define the two inputs of the FIS. This was because the bell type MF was unable to capture the dynamic information provided by the training data that were generated from the exact dynamic model of the motor. Initially the training data were generated the simple motor’s reference model.

- Initial Parameters of MFs

During the early development of the FIS, where it was initially developed on the simple reference motor model, ANFIS training was carried out using the initial parameters of the input MFs that were predefined by the FIS editor. By default, the FIS editor would spread all the MFs equally across the specified range (the universe of discourse). However, during one of the ANFIS training, the input MFs were initialised differently from its usual ‘spread’, and it was observed that the resulting training performance improved vastly over the previous FIS that had been trained with the default ‘spread’. This led to the next work of verifying that observation by initialising three FIS for training, each of which the input MFs have been distributed differently across the FIS input range. These three different methods were given the names Spread, Wide and Narrow, which have been defined graphically in Fig. 10 (a), (b) and (c), respectively. It may have been observed that seven bell MFs were used to define the input variable instead of five. This was because at that point of time, it was yet to find the effectiveness of using five MFs. Fig. 10 (i), (ii) and (iii) correspond to the resulting speed control performance from using Spread, Wide and Narrow initialisation.

Degree of Membership (DOM)

Speed Error, e (rad/s)

Speed Error, e (rad/s)

Degree of Membership (DOM)

Speed Error, e (rad/s)

Degree of Membership (DOM)

Speed, N (rpm)

Time, t (s)

Speed, N (rpm)

Time, t (s)

Speed, N (rpm)

Time, t (s)

(a)                                                                                                 (i)

(b)						                   (ii)

(c)						                       (iii)

Fig. 10: Comparing the speed control performance between different initializations of the input MFs’ parameters (defined as Spread, Wide and Narrow initializations) at random step speed commands of 6000 rpm. (a), (b) and (c), respectively, represent the initial arrangement, or distribution, of the input bell type MFs for Spread, Wide and Narrow before training. (i), (ii) and (iii), respectively, represent the resulting speed control performance of a Spread-, Wide- and Narrow- initialized FIS after training.

These results clearly indicate the influence of the manner in which the parameters of the input MFs are initialised has on the training performance. In this case, the ‘narrow’ initialisation of the input MFs has the best performance as it does not require any gain adjustment or bias removal. However, the final developed FTAG FIS algorithm employed the ‘spread’ initialisation for training because both the triangle and trapezoidal MFs could not be trained when it was initialised using the ‘narrow’ method.

- Number of Input Variables and Rules

During the initial development stage of the FIS, it was initialised to have one input and one output for training. At that time, the one input FIS that had the best speed control performance was the trained FIS that had five bell type input MFs, and thus, five rules, and trained with 1

rpm range data. The only obvious problem with that FIS was its poor performance during load changes. This is due to high percentage of over and undershoots. Hence, an extra input variable named Change in Speed Error, CE (rad/s), was added to determine whether increasing the number of input variables to the FIS would improve its performance from that aspect. Besides, most papers such as [9-11, 15-16] talk about FIS that has two and not one input, especially those that are used as motor speed controllers. The 1 rpm training data for the second input of the FIS was generated from the reference motor model. As the ANFIS Editor can only train a FIS that has a minimum of two MFs per input, the second input was defined by three bell type MFs. The 15 rules base that was used is shown in Table 1.

| CE (rad/s) E (rad/s) | ep < e | ep = e | ep > e |
| --- | --- | --- | --- |
| wm >> wm* | GR1 | GR2 | GR3 |
| wm > wm* | R1 | R2 | R3 |
| wm = wm* | NC1 | NC2 | NC3 |
| wm < wm* | I1 | I2 | I3 |
| wm << wm* | GI1 | GI2 | GI3 |

Table 1. The rule base that was used for the two input FIS, rpm1_input2.fis. wm: actual motor speed, wm*: desired motor speed, ep: previous speed error, e: current speed error, GR: Greatly Reduce, R: Reduce, NC: No Change, I: Increase, GI: Greatly Increase.

As an example, one rule is considered from the table:

IF wm is very much slower than wm*, i.e. e is wm << wm*,

AND ep = e, THEN let Iq* = GI2, so that the actual motor speed can catch up with the command speed.

The reason 15 different output linear MFs were used to define the output was because the number of output MFs must be equal to the number of rules in the FIS rule base when using the MATLAB ANFIS Editor to train. That is the reason why we could not use the same output linear MF to represent G1, G2 and G3 as G, or I1, I2 and I3 as I, and so forth, such that the number of output MFs can be reduced to just five- GR, R, NC, I and GI.

As for the initialisation of the Sugeno output linear MFs, its influence on the training was verified by comparing the resulting speed control performance of two FIS; one FIS whose output linear function’s constants were all initialised to zero whereas in the second FIS, the constants were predefined and ‘equally’ distributed by the FIS editor. From this comparison, it was found that the initial parameters of the output MFs make no difference to the resulting performance of the trained FIS; the resulting trained FIS parameters are identical to one another.

A possible reason why the initial parameters of the output MFs does not have as much influence on the ANFIS training (performance) as the input MFs would be due to the fact that during an ANFIS training, the Sugeno output MFs are trained first before the parameters of the input MFs are adjusted [1,6]. Hence, regardless of the initialisation of the Sugeno output MFs, the training algorithm will adjust its parameters first, until either the specified error tolerance or number of training epochs has been reached. However, how well the output MFs are trained depends on the manner in which its input MFs have been initialised.

From Fig. 11, it can be seen that the FIS that has two inputs has better speed control performance during step load changes, even at a speed as low as 1rpm. It was also verified that as the speed command increases, the performance of the two input FIS improves further. However, at higher speeds, the performance of the one input and two input FIS are similar.

- Training Method

The ANFIS Editor uses either backpropagation or a combination of LSE and BP for MF parameter estimation. One is not necessarily better than the other as it is dependent on the nature of the training data that the FIS is being trained to map. Throughout the development of the adaptive speed control algorithm, the hybrid FIS model parameter optimisation method was employed. A comparison between the two optimisation methods were not carried out. As for the choice of the training error tolerance, it was left at the default zero value. However, a quick test was carried out to determine whether there was a difference between the resulting training performance after a FIS has been trained for just 5 epochs and then for 20 epochs. It was found that the FIS does not need to be trained anymore than it takes for its training error to converge to a constant. Hence, if the training error converges from the second epoch onwards, the FIS does not need to be trained anymore than that.

Speed, N (rpm)

Time, t (s)

Fig. 11. Comparing the speed control performance between a 2 input FIS and a 1 input FIS at a low speed of 1 rpm. A fixed input gain of 20 was used in both cases.

### 4. 	Conclusion

This paper has presented the results of an extensive comparative study related to the ANFIS parameters that are involved in the development of an adaptive FLC for high performance ac drives. The ANFIS parameters encompassed by the study include the type, number, distribution and range of the training data that is most suitable to be used to train the FIS during the ANFIS training, initialisation of the FIS architecture i.e. the number of input MFs, the type of Sugeno inference model (Zero or First Order method), type and initial parameters of the MFs, and the number of input variables and rules, that will facilitate the FIS to learn efficiently and effectively, and lastly, the training method that is most appropriate to be employed to train the FIS. The transients are examined for both high (at rated speed) and low (<1% rated speed) speeds commands, as well as at the motor’s FL and NL condition, thus enabling a thorough analysis to be made. The main conclusions may be summarized as follows:

- The training data should contain sufficient information that will represent the sort of data that the FLC will be facing during its operation on the motor, such that the ANFIS trained FLC will exhibit the best response to the data it receives at its input. However, there should be balance between the number of data and the number of FIS parameters being adjusted by ANFIS, which corresponds to the number of MFs being utilized.

- The initialisation of the FIS architecture is an important element in the training as it determines how efficiently it can capture the information stored on the training data. Again, determining which FIS architecture works best for the application depends on the complexity of the nature of the training data that it is trying to learn from.

- The training does not need to be carried out any longer that it needs for the training error to converge. The faster the error converge, the better is the learning ability of the FIS being trained.

References

1. 	B. K. Bose, Modern Power Electronics and AC Drives, Upper Saddle River, NJ: Prentice- Hall PTR, 2002.

2. 	M. N. Uddin and M. A. Rahman, “ Fuzzy Logic Based Speed Control of an IPM Synchronous Motor Drive”, Proceedings of the 1999 IEEE Canadian Conference on Electrical and Computer Engineering Shaw Conference Centre, Edmonton, Alberta, Canada, May 9-12 1999, pp. 1259-1264.

3. 	M. Azizur Rahman, M. Nasir Uddin and M. A. Abido, “ Development and Implementation of a Hybrid Intelligent Controller for Interior Permanent- Magnet Synchronous Motor Drives”, IEEE Transactions on Industry Applications, Vol. 40, No. 1, January/ February 2004, pp. 68-76.

4.	Brian Heber, Longya Xu and Yifan Tang, “ Fuzzy Logic Enhanced Speed Control of an Indirect Field- Oriented Induction Machine Drive”, IEEE Transactions on Power Electronics, Vol. 12, No. 5, September 1997, pp. 772-778.

5.	Li Zhen and Longya Xu, “ Fuzzy Learning Enhanced Speed Control of an Indirect Field- Oriented Induction Machine Drive”, IEEE Transactions on Control Systems Technology, Vol. 8, No. 5, March 2000, pp. 270-278.

6.	Michael Negnevitsky, Artificial Intelligence: A Guide to Intelligent Systems, second edition, Addison Wesley, Pearson Education Limited, 2005.

7. C. M. Kwan and F. L. Lewis, “ Robust Backstepping Control of Induction Motors Using Neural Networks”, IEEE Transactions on Neural Networks, Vol. 11, No. 5, September 2000, pp. 1178-1187.

8. I.-C. Baik, K.-H. Kim and M.-J. Youn, “ Robust nonlinear speed control of PM synchronous motor using adaptive and sliding mode control”, IEEE Proceedings on Electrical Power Applications, Vol. 145, No. 4, July 1998, pp. 369-376.

9.	Ali Zilouchian and Mo Jamshidi, Intelligent Control Systems Using Soft Computing Methodologies, Boca Raton, FL: CRC Press, 2001.

10.	Chin- Teng Lin and C. S. George, NEURAL FUZZY SYSTEMS: A Neuro- Fuzzy Synergism to Intelligent Systems, Upper Saddle River, NJ: Prentice- Hall PTR, 1996.

11.	Math Works Inc., Neural Network Toolbox For Use with Matlab® User’s Guide Version 4, 2004.

12.	Math Works Inc., Fuzzy Logic Toolbox For Use with Matlab® User’s Guide Version 2, 1998.

13. 	Han- Xiong Li, “Adaptive Fuzzy Control”, IEEE, 1996, pp. 366-371.

14. 	K. Kouzi, L. Mokrani and M- S. Nait- Said, “ A Fuzzy Logic Controller with Fuzzy Adapted Gains Based on Indirect Vector Control for Induction Motor Drive”, Journal of Electrical Engineering, Vol. 3, 2003.

15. 	Z. Ibrahim and E. Levi, “ A Comparative Analysis of Fuzzy Logic Speed Control in High Performance AC Drives Using Experimental Approach”, IEEE, 2000, pp. 1217-1224.

16. University of South Australia, Neural Networks COMP 4009 Course Information Student Booklet Sem1 2004, Division of Information Technology Engineering & Environment, School of Computer and Information Science, 2004.

17.	C. M. Kwan and F. L. Lewis, “ Robust Backstepping Control of Induction Motors Using Neural Networks”, IEEE Transactions on Neural Networks, Vol. 11, No. 5, September 2000, pp. 1178-1187.

### 18.	M. Nour I. Aris, N. Mariun, S. Mahmod, “Hybrid Model Reference Adaptive Speed Control for Vector Controlled PMSM Drive”.  IEEE Sixth International Conference on Power Electronics and Drive Systems (PEDS05), Nov. 2005, K. L., Malaysia, pp.618-623.

19. T. S. Radwan, M. M. Gouda,“ Intelligent Speed Control of Permanent Magnet Synchronous Motor Drive Based-on Neuro-Fuzzy Approach”, IEEE Sixth International Conference on Power Electronics and Drive Systems (PEDS05), Nov. 2005, K. L., Malaysia, pp.602-606.